ATTACHMENT 3 Rev 0
PROBE ZERO OF THE STRESS Tel T-Mike Programmable

ULTRASONIC THICKNESS TESTER

How to Probe Zero

- Press the ON/OFF button to turn the unit on.

- Apply a drop of couplant to the end of the transducer.

- Place the transducer in steady contact probe disk located on the top of the T-Mike.

- The T-Mike should recognize the probe and zero the probe.  Pro 0 should be displayed.

- Take thickness readings and press the ON/OFF button to turn the unit off.

How to Change Velocities

The T-Mike is programed with the velocity for steel. It has to be connected to a computer with software installed that allows for the velocity to be changed.  Contact your supervisor if it is necessary to measure the thickness of materials other than steel.